# Infonique iSEB Expansion Board

# 1200 0050

| Prepared by | Date | Version |
| --- | --- | --- |
| Bing Ran | 14/10/2024 | 1.0 |

# Abstract

This document provides detailed of Infonique expansion board 1200 0050 specification. We will mention it as expansion board from now onward.

# Document History

| Date | Rev | Modifier | Changes |
| --- | --- | --- | --- |
| 14-Oct-2024 | 1.0 | Bing Ran | First Draft |

# Contents

Abstract	2

Document History	2

Contents	3

Table of Figures	4

Table of table	4

1 Introduction	5

1.1 PINOUT 	5

1.2 Connectors	6

1.2.1 Power Input	6

1.2.2 Motor Connectors	6

1.2.3 Button	7

1.2.4 Infrared Receiver	7

1.2.5 RBG LED Connectors	8

1.2.6 Buzzer	8

1.2.7 Ultrasonic Sensor Connector	9

1.2.8 Bluetooth Connector	9

1.2.9 OLED Display Connector	9

1.2.9 AI Husky Lens Connector	10

1.2.10 IR Tracking Sensor Connector	10

# Table of Figures

Figure 1  Top and bottom layer of expansion board	5

Figure 2  Power Connector	6

Figure 3  Motor Connectors	6

Figure 4  Button	7

Figure 5  Infrared Receiver	7

Figure 6  Example of infrared remote control	7

Figure 7  RBG LED	8

Figure 8  Example of round and strip LED	8

Figure 9  Buzzer	8

Figure 10  Ultrasonic Sensor Connector and HC-SR04 ultrasonic sensor	9

Figure 11  Bluetooth connector and HC-05 bluetooth module	9

Figure 12  OLED display connector and OLED display	9

Figure 13  AI Husky Lens Connector	10

Figure 14  Connector for IR tracking sensor	10

# Table of table

Table 1  Expansion board pinout	5

# 1 Introduction

Infonique Expansion board to plug onto the Arduino UNO board. It operating voltage is base on Arduino UNO. It is about  5.7cm(L) x 5.3cm(W). It ables to drive

Figure 1 Top and bottom layer of expansion board

## 1.1 PINOUT

| Pin | Function | Pin | Function |
| --- | --- | --- | --- |
| D0 | UART Rx | D10 | Right Motor Encoder A |
| D1 | UART Tx | D11 | Buzzer |
| D2 | Infrared sensor Rx/ IR Tracking Sensor A5 | D12 | Ultrasonic Echo |
| D3 | Right Motor Speed | D13 | Ultrasonic Trigger |
| D4 | Right Motor Encoder B | A0 | Left Motor Direction |
| D5 | Left Motor Speed | A1 | Right Motor Direction |
| D6 | Left Motor Encoder B | A2 | Button Input / IR Tracking Sensor A1 |
| D7 | Left Motor Encoder A | A3 | IR Tracking Sensor A2 |
| D8 | Bluetooth Tx | A4 | I2C SDA/IR Tracking Sensor A3 |
| D9 | Bluetooth Rx | A5 | I2C SCL/IR Tracking Sensor A4 |

Table 1 Expansion board pinout

# 1.2 Connectors

## 1.2.1 Power Input

- Expansion board provide connector for batter.

- The input voltage limit base on Arduino board voltage regulator.

- There are positive and negative sign on the connector.

- The board will be damaged if connected wrongly.

- The on board pcb connector is JST B2B-XH-A.

- The mating connecter will be JST XHP-2.

Figure 2 Power Connector

## 1.2.2 Motor Connectors

- Expansion board provides 2 motor connectors which are  right and left.

- It allows users to control 2 encoded DC motors simultaneously with motor drivers.

- It able to control two groups of speeds left and right.

- It able to control the direction of each motor.

- The on board pcb connector is JST B6B-XH-A.

- The mating connecter will be JST XHP-6.

Figure 3 Motor Connectors

| Number | Pin |
| --- | --- |
| 1 | M- |
| 2 | 5v |
| 3 | B |
| 4 | A |
| 5 | GND |
| 6 | M+ |

Table 2: Pinout arragement for motor connector

Figure 4: Connect JST B6B-XH-A

## 1.2.3 Button

- Expansion board provide a button for user to explore digital input.

- Users have to short the jumper with 2.54mm jumper cap in order to work with the button.

- The button come with a pull up resistor internal pull up is not required.

- The figure below is showing the jumper and button.

Figure 5 Button

## 1.2.4 Infrared Receiver

- Manufacturer : Vishay Part Number : HS0038 Operating Frequency : 38kHz

- Able to receive the signal from infrared remote control.

- Users have to short the jumper with 2.54mm jumper cap in order to work with the button.

Figure 6 Infrared Receiver

Figure 7 Example of infrared remote control

## 1.2.5 Buzzer

- Passive buzzer able to have different tone with change the frequency of the PWM signal.

Figure 8 Buzzer

## 1.2.7 Ultrasonic Sensor Connector

- Able to work with HC-SR04 ultrasonic sensor.

Figure 9 Ultrasonic Sensor Connector and HC-SR04 ultrasonic sensor

## 1.2.8 Bluetooth Connector

- Able to work with HC-05 Bluetooth module through serial communication.

Figure 10 Bluetooth connector and HC-05 bluetooth module

## 1.2.9 OLED Display Connector

- Able to work with SSD1306 driver OLED display module  through I2C communication.

Figure 11 OLED display connector and OLED display

## 1.2.10 IR Tracking Sensor Connector

- Able to work with IR Tracking Sensor with A2,A3,A4,A5 ,5v and GND pin.

- IR receiver ,button and OLED display cannot work simultaneously with IR tracking sensor because they are using the same pins with IR tracking sensor.

- The remove the jumper cap for button and IR receiver is need in order to use IR tracking sensor

- The connector provided for IR tracking sensor is 2.54mm 6pin male pin.

Figure 12 Connector for IR tracking sensor

| Number | Pin |
| --- | --- |
| 1 | Gnd |
| 2 | Vcc |
| 3 | Out1 |
| 4 | Out2 |
| 5 | Out3 |
| 6 | Out4 |
| 7 | Out5 |

Table 3: Pin arragement for IR tracking sensor from left to right